# Dxf2Et - the DXF-Converter for EdiTasc

Versions 2.xx.xx

This manual and the accompanying software are proprietary to TRIMETA software.

Copyright © 2000-2005 by TRIMETA software GmbH. All rights reserved.

The referenced trademarks are property of their respective holders.

Contents

Dxf2Et - the DXF-Converter for EdiTasc	2

Features of Dxf2ET	3

Evaluation of DXF-ENTITIES	4

System requirements	4

The Main Window	5

Overview	Fehler! Textmarke nicht definiert.

The Menu Bar	6

The Toolbar	8

The Tree View	9

The Graphic Pane	10

The Right Pane	11

Auxiliary Windows	13

The Pocket Window	14

The Offset Window	14

Tutorial	16

Optimizing the processing order	17

Rules for Reordering	17

Tutorial	19

Short Summary	23

Glossary	25

## Features of Dxf2ET

The Dxf-Converter converts DXF files to moving commands for EdiTasc. The DXF-objects (lines, arcs and circles) are displayed graphically and can be manipulated concerning order, orientation and tool selection.

New: The graphic output is generated with genuine EdiTasc code so that the milling paths are exactly the same when machining takes place.

The output of the converter consists of macro calls for each object found in the DXF file along with commands to allow cutter radius compensation and to generate drill holes and pockets.

### Details:

The Dxf-Converter reads the entities "LINE", "ARC", "CIRCLE", "POLYLINE" and "LWPOLYLINE" as well as the block structure and the information contained in insert blocks. The structure of the input file can be displayed with an extra tree view. The informations within "HEADER", "OBJECT" and  "TABLES" sections are ignored.

Single objects and groups of objects can be selected and grouped for output. For each object (except circles) the orientation can be changed. Connected paths can be selected, orientated and grouped for treating with special tools. It is possible to generate pockets from circles and rectangles.

The output of the converter can be reopened to change settings like tool selection, cutter radius compensation and even to undo and remake pocket definitions.

### Evaluation of DXF-SECTIONS:

| HEADER | Scale factors are extracted |
| --- | --- |
| CLASSES | ignored |
| TABLES | ignored |
| BLOCKS | evaluated |
| ENTITIES | evaluated |
| OBJECTS | ignored |

### Evaluation of DXF-ENTITIES

| LINE | straight line |
| --- | --- |
| CIRC | circle |
| ARC | circular arc |
| LWPOLYLINE | short form of polygon |
| POLYLINE | polygon with lines and arcs |
| VERTEX | where parts of a polyline are connected |
| SEQEND | closes the definition of a polyline |
| VIEWPORT | is not supported |

### System requirements

The converter is designed for WINDOWS 2000/WINDDOWS XP.

# The Main Window

When Dxf2Et is started, the following elements are on the screen:

(1) A menu bar with entries for file handling and options.

(2) A tool bar with command buttons to invoke program action.

(3) A tree view to show details of the loaded file.

(4) A graphic pane to show the graphical representation of the loaded objects.

(5) A right pane with additional buttons to maintain the program and to save the result of the conversion.

The milling process and the graphical representation is controlled by macros. The converter generates only the macro calls and allows therefore to modify the processing of objects by editing the macro code.

## The Menu Bar

### The File Menu

| Command | Action |
| --- | --- |
| Open | Opens files with DXF- or EdiTasc-format. |
| Save | Saves the loaded file with .T- format in the current folder. |
| Save as | Allows to save the loaded file with .T-Format in a different folder. |
| Close | Closes the converter. |

### The Options Menu

| Commands | Actions |
| --- | --- |
| Delete | Delete non relevant objects from tree |
| Arrows | Arrows are drawn to show the processing direction |
| Show transition periods | Shows/hides the transition periods |
| Message box before writing | If checked, you will be prompted for writing the output file |

## The Toolbar

| Command | Action |
| --- | --- |
| Select | When this button is pressed, you can select objects by clicking on the graphic pane or on the tree view. If the object is already selected, it will be deselected. Selected objects are shown in green color. |
| SelGroup | When this button is pressed, you can select objects by clicking on the graphic pane. If an object is found, all objects within the same group are selected too. No deselection takes place. |
| SelBox | When this button is pressed, you can select objects by dragging a box with the mouse on the graphic pane. All objects which are entirely in the box will be selected. |
| Revert | Revert allows to change the orientation of an object. |
| Connect | When this button is pressed, you can to pick up connected lines and arcs by clicking on one object with the mouse. Two objects are "Connected", when the gap between adjacent points is less than "MaxGap". The value of MaxGap is read from the Dxf2Et.ini file in section [Settings] with 0.05 mm as default. The connected objects will be selected and oriented according to the originally selected object. |
| Pocket | This button allows the generation of circular and rectangular pockets. The parameters of the pocket (depth, number of levels) will be asked by an extra window. The recognition of rectangles depends on "MaxGap", see "Connect". |
| Offset | This button allows do assign an offset to one selected object. This offset will be applied on conversion to all objects in that way, that the starting point of the selected object has predefined coordinates. An extra window will open to ask for the predefined coordinates. The starting point of the selected object will be marked with a red circle while the window is open. |
| Clear | “Clear" resets the current selected objects and clears all pending actions. |
|  | "Zoom in" allows to draw a rectangle which will be enlarged to fit the graphic pane. |
|  | "Zoom back" goes one step back in the zoom history. |
|  | The objects will be enlarged to fit the graphic pane. |

## The Tree View

The tree view shows the objects along with the selected process types and tools.

The header entry (1) shows the input file name.

If a DXF file has been loaded, the objects are collected in the "Section ENTITIES" (2). Above this line the block definitions (3) of the input file can be shown if any. The graphic pane switches from block view to entities view when an appropriate line of the tree is clicked.

The deepest level of the tree view shows the coordinates of the objects (4). The symbol shows the object type (line, arc, circle, pocket). If an object is selected, the symbol has a green background color.

Consecutive objects of same type are grouped in "Collections" (5). A Collection shows the number of objects within that collection. The symbol shows the machining type.

All collections with same machining type are members of a "Group" (6). The group may have an individual name and contains data about the machining type and the tool to work with. The symbol shows whether the group contains separate or connected objects. In case of connected objects the symbol shows if the objects constitute a closed shape. Each object is machined with the parameters of the smallest containing group.

Groups may be nested. The "Section ENTITIES" and the main entry are groups too. A Polyline form DXF input is converted to a group.

INSERT entries (7) are also groups and contain copies from members of the "Section BLOCKS". They behave like other groups.

#### You may make the following changes:

1) By single mouse click:

- Select/deselect of objects, when the "select"-button is pressed. The selection is toggled. Selected objects are drawn with green color.

2) By double-click:

- Change the name of the groups. This will not work with the top groups because their names are write protected.

3) By single click with the right mouse button (a context menu pops up):

- Change the machining type of a group.

- Change the tool number of a group.

- Show/Hide a group.

- Select/deselect a collection or an object (no toggling takes place).

- Delete a group, a collection or an object.

- Test a group whether it is connected or closed.

4) By dragging a group, a collection or an object over some other part of the tree:

- Change/optimize the order of objects.

The contents of a block cannot be modified in this version.

## The Graphic Pane

The graphic pane shows the objects to be machined. According to the setting for "colors" the objects are shown by machining type (default) or by tool number.

## The Right Pane

|  | Frame/Button Objects Color from Group Save as... | Meaning/Action The number of objects within the tree categorized by object type along with total the number of nodes. Object colors can be taken form machining type or from tool number. To generate a new group from all currently selected objects. Generates the output file in MTasc format for machining with EdiTasc or in HPGL format |
| --- | --- | --- |

# Auxiliary Windows

## The Pocket Window

The pocket window opens when generating of a rectangle or circle pocket is selected.

The pocket will be milled in steps beginning from "Surface of object". Each step will increase the depth by "Thickness of layer". The last step will be adjusted to give the exact result of "Depth of pocket".

If "Smoothing" is selected, then the tool will be kept away form the border by the given distance. The finishing takes place either in one step or for each level separately.

## The Offset Window

By using the offset window you can move the whole machining process to a given point of the working area. This can be achieved if you know the coordinates of one object in relation to the current origin.

The offset window appears when "Offset" is selected from the toolbar and then an object on the graphic pane is selected with the mouse. The starting point of the selected object will be marked with a red circle (see white arrow).

The offset window will show the coordinates of the center of the red circle. The window allows to assign new values to this point.

When output is generated all coordinates are shifted in that way, that the selected point gets the coordinates from the offset window. If the output is read back to the converter again, the original coordinates will be lost unless the offset window is opened again with the same point and the original values are inputted there.

#### Note:

It is not recommended to select a pocket object for defining an offset with the offset window.

# Tutorial

## Optimizing the processing order

#### Collect Objects with same machining type / tool:

- Activate the "Select" button and select the objects by clicking into the graphic pane. The object next to the mouse pointer will be selected/deselected.

- Alternatively or additional you may activate the "SelGroup" button and click an object. The object will be selected along with all objects in the same group. No deselection takes place.

- Alternatively or additional you may activate the "SelBox" button and draw a rectangle on the graphic pane. All objects which are entirely in the box will be selected. No deselection takes place.

- Alternatively or additional you may select objects by clicking on the tree view. The selection of objects will be toggled.

- Alternatively or additional you may click on a collection or an object with the right mouse button. The context menu allows you to select/deselect the collection or the object respectively.

- Make a new group of all selected objects by clicking "Group" from the right pane. You may want to give the group a new name and select machining type and tool for this group.

#### Connect consecutive objects:

- Activate the "Connect" button and click an object of a connected contour. All adjacent lines and arcs are selected and connected to a contour with consistent orientation.

- By activating "Select" or "SelBox" you can add more objects to the selection.

- The selected objects can be grouped as described above.

#### Optimizing the order of objects within one group:

- Activate the "Select" button and select objects in the desired sequence. You can deselect a selected object by clicking it again. To reorder the objects, click the "group" button on the right pane. You may dissolve the group thereafter keeping the new order of objects.

- Alternatively you may open the group in the tree view and reorder the objects by dragging them on the desired place. To check which line corresponds to which object you can keep the "selec" button pressed an highlight the object by clicking the line.

## Rules for Reordering

The nodes of the tree view can be easily reordered by dragging them with the mouse to another place. We call the moved node the "source node", the node on which the source node is dropped is called the "target node".  For the result it is essential

- which type of node (object, collection or group) are source node and target node

- whether the target node is open or closed

- for object nodes and collection nodes, whether they represent the same object types (line, circle, arc) or not.

#### The main rule is:

- If the target node is a closed group node, no object will go into this node. To move objects into a group node, the node must be opened first.

- If an object node or a collection node is dropped over a closed collection, the source objects go into the collection if and only if the object types are the same. The objects are placed at the end of the target collection.

#### Rules for source object nodes:

- If the target is an object: The source object will be inserted after the target object. When the types are different, a new collection is generated and inserted after the target collection.

- If the target is an open collection (the objects are visible): The object will be inserted on the top of the target collection. When the types are different, a new collection is generated and inserted before the target collection.

- If the target is a  closed collection (the objects are not visible): The object will be appended to the target collection. If the types are different, a new collection is created and inserted after the target collection.

- If the target is an open group (the collections and objects are visible): The object will be inserted in the first collection of the target group. If the types are different, or if the first object in the target group is a nested group, a new collection is created and inserted on top of the target group.

- If the target is a closed group (no collections are visible): A new collection is generated and inserted after the target group.

#### Rules for source collection nodes:

- If the target is an object: If the source objects and the target object have the same type, the source objects will be inserted after the target object. If the types are different, the target collection is divided into two collections with the target object as last object of the first part. The source collection will then be inserted between the two parts.

- If the target is an open collection (the objects are visible): If the source objects and target objects have the same type, the source objects go to the top of the target collection. If the types are different, the source collection will be placed before the target collection.

- If the target is a  closed collection (the objects are not visible): If the source objects and target objects have the same type, the source objects are appended to the target collection. If the types are different, the source collection will be placed behind the target collection.

- If the target is an open group (the collections and objects are visible): The collection will be inserted on top of the group.

- If the target is a closed group (no collections are visible): The collection is inserted behind the group.

#### Rules for source group nodes:

- If the target is an object: Groups can not be dropped on objects.

- If the target is a collection:  The group will be placed behind the collection whether the target collection is open or not. If the target collection is open, a warning message is issued.

- If the target is an open group (the collections and objects are visible): The group will be inserted on top of the group.

- If the target is a closed group (no collections are visible): The group is inserted behind the group.

## Step by Step Example

The following sections show step by step the conversion process from DXF to MTasc. See short summary for a brief description.

Sections:

1. Prepare to Start

2. Prepare Objects

2.1 The Frame

2.2 The Upper Square

2.3 The Lower Square

2.4 The Drilling holes

3. Optimizing the Paths

3.1 Reordering Groups

3.2 Reordering Objects

4. Convert and Save

### 1. Prepare to Start

Open a file with DXF- or ET-format:

Select "Open" from the file menu or click "SELECT INPUT FILE" in the tree view.

A file selection window opens:

| - Select the format to be converted - Select the file to be converted - Click the "Open" command button |  |
| --- | --- |

The tree view shows all objects with initially set machining type to "draw" and tool=0. The objects are therefore drawn in white color. The shadowed lines show the transition paths:

### 2. Prepare Objects

Use the toolbar and the tree view.

#### 2.1 The Frame

First the objects which make the frame will be joined to a connected line:

Click the connect button, then an any object of the frame (line or arc). The objects will be connected and highlighted with green color.

Now rename the entry "Group to draw" by double clicking into "Frame". Then select machining type "milling" with right mouse click on "Frame to draw" along with some suitable tool. Click "Clear" to reset the toolbar.

The frame now appears in red color to indicate the machining type.

#### 2.2 The Upper Square

We want to mill the upper square from inside. From the milling direction (see arrows) we learn that the radius compensation must go to the left side of the path.

First, we change the group name "Group to draw" into "Upper Square". Then we click the group with the right mouse button and select "milling <-" from the Process Type menu. After this we choose a suitable tool with at least 4 mm radius. The square appears now in pink indicating the milling type. A yellow line inside the square shows the path of the tool.

In the left lower corner of the square we see that the milling path is not closed correctly. This is because the group is treated as "not connected". To change this behaviour, select "Close test" from the context menu of the group (right mouse-click) and then "Refresh" from the graphic pane. Now the path is perfectly closed. See next picture for the result. The additional pink line shows the "approach" of the tool to the starting point which has been moved to the right corner of the square.

#### 2.3 The Lower Square

From the lower square we create a pocket. Therefore click "Pocket" from the toolbar and then click on one of the borders of the square. Supply the pocket window with some suitable data.

To make tool selection possible, a containing group must be found. This is "Section ENTITIES" which contains objects to be machined with different tools. So we have to create a new group for the pocket:

Click "clear", then "Select", then the "Pocket" line in the tree view and finally "Group" in the right pane. After this the new generated group can be supplied with machining type "milling" and some tool.

After clicking "Clear" again the pocket appears with red milling paths.

#### 2.4 The Drilling Holes

We want the circles to be treated as drilling holes. As this is a machining type different from others, the circles must have their own group.

Clicking "Clear", "Select" and selecting the circles (within the tree view or the graphic pane) will do this. The circles are green after selection. Now click "Group" from the right pane.

Rename that group into "Drill Holes" and give them the appropriate machining type and a suitable tool to complete the process.

If you now click on "Tool" in the "Colors" frame, the milling paths are shown in colors with respect to the selected tools.

### 3. Optimizing the Paths

#### 3.1 Reordering the Groups

By dragging the group entries in the tree view you can choose the following sequence Frame - Upper Square - Pocket - Drill holes (see next picture). Clicking "Refresh" will eliminate superfluous transition paths.

#### 3.2 Reordering the Objects

You may want to optimize the transition paths between the drilling holes too. To do so, click "Select" and then click the circles in the desired order. "Group" will place the holes like this, thus obtaining a slightly shortened path.

Alternatively, you can drag the last circle on the top of the second one to achieve swapping the last two circles. Moving the third circle upon the last one does the same.

### 4. Convert and save

Conversion takes place when "MTasc" or "HPGL" is clicked. You may also use the file menu with Option "Save as..." to save the file to a different location. The conversion of the DXF file is now completed. In case of "MTasc" you can reload the generated file again to make changes.

## Short Summary

#### 1. Open DXF file with "Open" from file menu.

#### 2. Group objects (lines, arcs and circles):

- Create connected contours: - Click "Connect", then click a piece of the contour to be connected. The connected objects are moved into a new group.

- Change starting point of a closed contour: - Click "Connect", then select the piece of the contour to start with.

- Change orientation: - Single object: Click "Revert", then click object where you want to change orientation. - Connected contour: Click "Revert", then click one object, then click "Connect", then click the same object again.

- Group objects: - Use "Select", "SelGroup" or "SelBox", then click "Group" from the right pane: Objects will be grouped in the order of selection. - Alternatively change the order of objects by dragging lines within the tree view. The Rules for Reordering apply.

#### 3. Setup of Machining Type and Tool

- Click a group of the tree view with the right mouse button. A context menu allows to select a machining type.

- Click a group of the tree view with the right mouse button. A context menu allows to select a tool type.

#### 4. Conversion and Output to File

Conversion takes place when one of the "MTasc" or "HPGL" buttons on the right of the graphic pane is clicked. You will be prompted for the name of the converted file. The "HPGL" button is not available in some versions of EdiTasc.

# Glossary

### Arc

Arc is the DXF term for a piece of a circle. In DXF format an arcs are always oriented counter-clockwise. For clockwise milling the orientation of an arc must be changed.

### Circ

Circ is the DXF term for a circle. Circles can be selected for milling or for drilling.

### Circle Pocket

Circle pockets can be generated form circles by clicking "Pocket" from the toolbar and then clicking a circle. A "pocket window" opens to let you input the parameters for the pocket.

If the clicked object is already a pocket, then the pocket definition can be erased by confirming the following question with "yes".

### Collection

A collection contains one or more objects with same object type. Each collection belongs to a group which defines machining type an tool for the objects. The tree view shows the machining type as picture and the number of objects along with some descriptive text in one line. Machining type and tool can only supplied by the containing group. If groups are nested, the parameters of the smallest group which contains the collection are in effect.

### Entity

Entity is a DXF term for the objects to be processed.

### Group

A group contains collections of objects with same machining type and same tool. Groups may be nested. The tree view shows groups with a picture describing the connectedness of the contained objects and the group name along with a short description of the machining type. Group names can be edited in the tree view.

### Layer

Layer is the DXF term for a group of objects to be machined or drawn. The converter converts layers into groups.

### Line

Line is the DXF term for a straight line.

### LWPOLYLINE

Like Polyline, with reduced data (Light-Weight-PolyLine).

### Machining Type

"Type" describes the machining type for an object. Machining types may be "drawing", "milling", "drilling". Milling can be with or without cutter radius compensation. Drilling can be horizontal or vertical.

| Type | Meaning |
| --- | --- |
| 0 | Draw only |
| 1 | Milling without cutter radius compensation |
| 2 | Milling with cutter radius compensation to the right |
| 3 | Milling with cutter radius compensation to the left |
| 4 | Vertical drilling |

### Object

Objects are the smallest units to be manipulated with the converter. In this version, the converter can maintain the following objects:

- The straight Line

- The Circle

- The (circular) Arc

- The rectangular pocket (RPoc)

- The circular pocket (CPoc)

Objects are always contained in a collection of (on or more) similar objects.

### Polyline

Polyline is a DXF term for a continuous path consisting of lines and arcs. The arcs of a polyline are defined by starting and ending point of a line along with a parameter "bulge".

### Rectangle Pocket

Rectangle pockets can be generated form rectangles by clicking "Pocket" from the toolbar and then clicking one of the borders of the rectangle. A "pocket window" opens to let you input the parameters for the pocket.

If the clicked object is already a pocket, then the pocket definition can be erased by confirming the following question with "yes".

The MTasc output for a rectangle pocket is a macro call with a diagonal line as parameter.

### Section

A DXF file contains several sections like "Header", "Classes", "Tables", "Blocks", "Entities", "Objects". The ENTITIES-Section contains the objects to be machined.

### Tool

"Tool" is the number of the tool with which an object is machined. The tool parameters are normally stored in the file "EtTools.etl". This file is used also by EdiTasc.